ชื่อ-สกุล..............................................................รหัส ............................. เลขที่ ............

- A gas is compressed from the initial volume of 0.42 m3 to the final volume of 0.12 m3. During the quasi-equilibrium, the pressure change with the volume according to the relation P = aV + b, where V is in m3, a = -1200 kPa/m3 and b = 600 kPa.

  - Sketch the graph of P-V diagram

  - What is the meaning of area under the P-V relation?

  - Using integrating method, calculate the amount of work during this compression process.

2 Mark

V

P

P1

V2

V1

2

1

P2

P = a+bV

Gas

V1 = 0.42 m3

2

a = -1200 kPa/m3

b= 600 kPa

1

V2 = 0.12 m3

2 Mark

1 Mark

System: gas, Closed system

Assumption: Ideal Gas, neglect  ΔKE,   ΔPE

Solution

1 Mark

(b) Boundary work: W = ∫PdV

Therefore,  Area under P-V diagram represent 1W2           answer

(c)      	1W2  = ∫PdV =   ∫(aV+b)dV

=	  a/2[V22-V12] + b[V2-V1]

a/2[V22-V12]    =   (-1200 kPa/m3)/2 x [0.122 – 0.422]   =   97.2 kJ

b[V2-V1]         =    (600 kPa)( 0.12 – 0.42)                    =  - 180 kJ

3 Mark

1W2              =  97.2 kJ – 180 kJ          = -82.8 kJ

The work done on the system is  82.8 kJ     Answer

1 Mark

ชื่อ-สกุล..............................................................รหัส ............................. เลขที่ ............

2)    Air of amount 2 kg in a rigid container 1.5 m3 receives heat rate at the amount of 0.5 kW for the period of  30 minutes. The gas constant R = 0.2870 kJ/kg•K and the   CP = 1.005 kJ/kg•K which are constant. The heating process starts with the temperature of the air at T1 = 27 ˚C. Determine

a)   The final temperature (in the unit of ˚C).

b)   The value of final pressure (in the form of gage pressure, Pa) if Patm = 1 bar.

m = 2 kg

V = 1.5 m3

T1 =27 oC

T2 =?

P2g = ?

Qdot = 0.5 kW

Time = 30 min

2

1

1 Mark

1 Mark

System: Air, Closed system, Constant volume

Assumption: Ideal gas

Analysis:

1st Law:                      1Q2 = 1W2 + m(u2-u1),    1W2 = 0

1Q2  =  mCv(T2-T1)

2 Mark

Relation:          Cp - Cv  = R

T2    =  T1 +  1Q2/mCv                       (1)

Heat added,     1Q2 =  Qdot x t                                   (2)

2 Mark

State 2:	          P2V =  mRT2                                     (3)

P2g   = P2 – Patm                                  (4)

Solution:

1Q2 =  Qdot x t   	=   (0.5 kW)( 30 mim)(60 s/min)

=          900 kJ

Cv  	=   Cp – R    =  (1.005 – 0.287)kJ/kgK  = 0.718 kJ/kgK

eqn (1),

1Q2/mCv       =    (900 kJ)/(2kg x 0.718 kJ/kgK)   =  626.7 K

T2             =  T1 +  1Q2/mCp    =  (27 oC +273) +  626.7 K      =  926.7 K

2 Mark

=	653.7 oC                                     Answer

P2=  mRT2 / V   =     (2kg x 0.287 kJ/kgK x 926.7 K) / (1.5 m3)

=   354.7 kPa

=    3.547 Bar

P2g   = P2 – Patm =    (3.547 – 1.0) Bar    =  2.547 Bar

=  254.7 kPa       Answer

2 Mark

ชื่อ-สกุล..............................................................รหัส ............................. เลขที่ ............

3. A piston-cylinder device contains steam initially at 1 MPa, 350oC, and 1.5 m3. Steam is allowed to cool at constant pressure until one-third of the mass of the steam condensed. Show the process on a T-v diagram with respect to saturation lines (1 mark) and determine,

(a) the mass of the steam  			(1 marks)

(b) the quality (x) at the final state, 		(1 mark)

(c) the amount of work during the process, 	(3 marks)

(d) the amount of heat transfer.			(3 marks)

(การทำงานอย่างเป็นลำดับขั้นตอนที่ดี การเขียนอย่างเรียบร้อยดูง่ายและสะอาด  1 คะแนน)

1 Mark

System	Water in the Piston-Cylinder:  Closed System:  P= const

Assumption 	Neglect change in KE, PE

Analysis		m 	= V1/v1

-1Q2  until ½ m  condensed  mixture: x2 = mvap/m

1W2   =  mP1(v2 - v1)

1Q2 	=U2-U1 +1W2

Solution

(a) Find mass of the steam:

State 1 	P1 = 1 MPa, T1 = 350 oC,  Table A-5 T1 > Tsat --> Superheated vapor

Table A-6	v1  = 0.2825  m3/kg;	   u1  =  2,875.2  kJ/kg

V1  =  1.5 m3

1 Mark

m   = V1/v1 =  (1.5 m3)/(0.2825m3/kg)

the mass of steam  is  5.31  kg	Answer

(b) Find the steam quality

1 Mark

State 2	P2 = P1 = 1 MPa and  1/3m condensed --> mvap= 1/3m

therefore; quality, x 	=  mvap/m 	= 2/3 = 0.6667	Answer

(c) Find Work : Boundary work: Process P =c ;	1W2  =  	mP1(v2 - v1)

Table A-5	vf  = 0.001127 m3/kg;   vg = 0.19444	 m3/kg

v2   =  vf + xvfg  = (0.001127 + 0.6667x 0.19444) =  0.1300 m3/kg

3 Mark

1W2  =  (5.31kg)(1000 kPa)(0.1300 - 0.2825) m3/kg  =  -809.7 	kJ

The work done on the system during the process is  809.7 kJ   Answer

(d) Find Heat

1st Law:		1Q2 = m(u2-u1) +1W2

Table A-5 at 1 MPa	uf  = 761.68	kJ/kg;   ufg = 1,822.0 kJ/kg

u2 = uf  + xufg	  = (761.88 + 0.6667 x 1,822.0)  = 1,976.3  kJ/kg

m(u2-u1)	 = 	(5.31kg)(1,976.3 - 2,875.2) kJ/kg   = 	-4,772.7 kJ

3 Mark

then,	1Q2 = 	(-4,772.7 kJ) + (-809.7 kJ)    = -5,582.4 kJ

The amount of heat transfer out of the steam is 5,582.4  kJ  Answer

1 Mark

Sommai Priprem

ชื่อ-สกุล..............................................................รหัส ............................. เลขที่ ............

4. Steam flows steadily through a turbine. The inlet conditions of the steam are 15 MPa, 650°C, and 200 m/s, and the exit conditions are 10 kPa, 90 percent quality, and 50 m/s. The mass flow rate of the steam is 150 kg/s. Heat loss from the turbine is 15 kJ/kg steam. State any assumptions made. Show the process on a T-v diagram with respect to saturation line and determine

(a)  the change in kinetic energy,

(b) the power output of the turbine.

Wcv

Turbine

1

2

P1 = 15 MPa

T1 = 650 C

V1 = 200 m/s

m = 150 kg/s

P2 = 10 kPa

x2 = 90%

V2 = 50 m/s

ΔKE = ?

Wcv = ?

q = 15 kJ/kg

Solution

Assumption: SSSF process,  ΔPE = 0

1 Mark

Analysis:

ΔKE = m(ke2-ke1)	     (1)

1st law: q + h1 + ke1 = w + h2 + ke2

1 Mark

w = q + (h1-h2) - (ke1-he2)    (2)

W = mw                                   (3)

Property:

State 1:	P1 = 15 MPa, T1 = 650 C

T > Tsat --> superheat

1 Mark

Table A-6, 	v1 	=  	0.0268 m3/kg

2 Mark

h1 	=  	3,712.30  kJ/kg

P1

T

1

2

P2

v

v1

v2

State 2:	     P2 	= 	10 kPa, x2 = 90 %  --> mixture

Table A-5     vf  	= 	0.001452 m3/kg,

vg 	= 	0.018026 m3/kg

v2 	= 	vf + x vfg   	      = 0.0163686 m3/kg

1 Mark

hf 	= 	191.83 kJ/kg,  hfg  = 2392.8 kJ/kg

h2 	= 	hf + xhfg   	      = 2,345.35  kJ/kg

T-v diagram: 	State 1  superheated vapor at P1

State 2  Mixture at P2 and v2 < v1

(a)	 ΔKE    =   m(ke2-ke1)

(ke2-ke1)    = 1/2{(50m/s)^2 - (200m/s)^2)}/1000 = -18.75	kJ/kg

1 Mark

m      = 150 kg/s

ΔKE     = m(ke2-ke1) = (12 kg/s)(-1.95 kJ/kg) = -2,812.50	kW

Change in kinetic energy (decreased) = 2,812.5 	kW	Answer

(b) from eqn 2	w 	= 	q + (h1-h2) - (ke1-he2)

q 	=	-15	kJ/kg

(h1- h2)	=	3712.3 kJ/kg - 2345.35 kJ/kg

=	1367.0	kJ/kg

ke1-ke2    =	18.75	kJ/kg

w 	= 	(-15kJ/kg)+(1,367.0kJ/kg)-(-18.75kJ/kg)

=	1,370,7	kJ/kg

W      =  mw 	=	(150 kg/s)( 1,370,7kJ/kg)

2 Mark

=	   205,605 	 kW

Power output from the turbine is   205.6  MW  		Answer

1 Mark

Sommai Priprem, July 2012