ภาควิชาวิศวกรรมเครื่องกล

คณะวิศวกรรมศาสตร์  มหาวิทยาลัยขอนแก่น

ข้อสอบกลางภาค ประจำภาคปลาย ปีการศึกษา 2553

วิชา 185 232 Thermodynamics II

วันอังคารที่   27 ธันวาคม 2554				                               เวลา 08:30-11:30 น.

ผู้ออกและตรวจข้อสอบ: รศ.ดร.สมหมาย ปรีเปรม ผศ.ดร.จุฬาภรณ์ เบญจปิยะพร

คำสั่ง

- ให้เขียน ห้องสอบ-ที่นั่งสอบ-กลุ่ม-รหัส- ชื่อ-สกุล ลงในกระดาษข้อสอบทุกแผ่น

- ข้อสอบทั้งหมดมี 5 ข้อ จำนวน 6 แผ่นรวมทั้งแผ่นนี้ แต่ละข้อมีคะแนนไม่กัน ให้ทำทุกข้อ

- มีตาราง Thermodynamics แจกให้ ห้ามขีดเขียนในตารางโดยเด็ดขาด

- อนุญาตให้ใช้เครื่องคำนวณที่ไม่มีการบันทึกข้อมูลของรายวิชานี้ไว้

- ห้ามนำเอกสารใดๆออกจากห้องสอบ

- ให้ทำข้อสอบในแผ่นข้อสอบของข้อนั้นๆเท่านั้น (สามารถใช้ด้านหลังได้)

ห้องสอบ..............ที่นั่งสอบ ............กลุ่ม.......รหัส ...................... ชื่อ-สกุล....................................................................

| ข้อ | คะแนนเต็ม | คะแนน |
| --- | --- | --- |
| 1 | 10 |  |
| 2 | 10 |  |
| 3 | 10 |  |
| 4 | 10 |  |
| 5 | 10 |  |
| รวม | 50 |  |

ห้องสอบ..............ที่นั่งสอบ ............กลุ่ม.......รหัส ...................... ชื่อ-สกุล....................................................................

1) A heat engine receives heat from a source at 1500 K at a rate of 700 kJ/s, and it rejects the waste heat to a medium at 320 K. The measured power output of the heat engine is 320 kW, and the environment temperature is 25°C. Determine (a) the Availability of the heat source, (b) the reversible power of the heat engine, (c) the rate of irreversibility of the heat engine , and (d) the second-law efficiency of this heat engine.  (10 marks)

ห้องสอบ..............ที่นั่งสอบ ............กลุ่ม.......รหัส ...................... ชื่อ-สกุล....................................................................

2) Steam enters a turbine steadily at 20 MPa and 500oC at a rate of 75 kg/s and exit at 10 kPa and 90% quality. The power produced is 65 MW. The surrounding is at 100 kPa and 25oC. Changes in kinetic and potential energy are negligible. Determine

(a) the rate of heat loss,

Turbine

1

2

P1 = 20 MPa

T1 = 500oC

ms= 75 kg/s

P2 = 10 kPa

x2 = 90%

P0 = 100 kPa

T0 = 25oC

(b) the availability of the steam at the inlet and exit conditions,

(c) the maximum possible power output (the reversible work),

(d) the irreversibility, and

(e) the second law efficiency.      (10 marks)

System: Steam flow through a turbine. Open System

Assumption: SSSF, neglect ΔKE, ΔPE

property of steam at each state:

| State | 0 | 1 | 2 | Unit |
| --- | --- | --- | --- | --- |
| P | 100 kPa | 20 MPa | 10 kPa |  |
| T or x | 25 | 500 | 0.9 | C |
| hf |  |  | 191.83 | kJ/kg |
| hfg |  |  | 2392.8 | kJ/kg |
| h | 104.9 | 3,238.20 | 2,345.35 | kJ/kg |
| sf |  |  | 0.6493 | kJ/kg |
| sfg |  |  | 7.5009 | kJ/kgK |
| s | 0.3674 | 6.1401 | 7.40011 | kJ/kgK |

|  | formular |  | value | unit |  |
| --- | --- | --- | --- | --- | --- |
| mdot | given |  | 75 | kg/s |  |
| Wcv | given |  | 65 | MW |  |
|  |  |  | 65,000 | kW |  |
| w | W/m |  | 866.67 | kJ/kg |  |
|  | h1-h2 |  | 892.85 | kJ/kg |  |
| q | w -(h1-h2) |  | -26.18 | kJ/kg |  |
| (a) Qcv | mq |  | -1,963.7 | kW | Answer |
|  | formular |  | value | unit |  |
| h-ho |  | 3,133.3 | 2,240.5 | kJ/kg |  |
| s-so |  | 5.7727 | 7.03271 | kJ/kgK |  |
| To(s-so) |  | 1,720.3 | 2,095.7 | kJ/kg |  |
| x |  | 1,413.0 | 144.7 | kJ/kg | Answer |
| (b) X | mx | 105,978 | 10,853 | kW | Answer |
| (c).Wrev | X1-X2 |  | 95,125 | kW | Answer |
| (d) I | Wrev -Wcv |  | 30,125 | kW | Answer |
| (e) 2nd Eff | Wcv/Wrev |  | 68.33 | % | Answer |

ห้องสอบ..............ที่นั่งสอบ ............กลุ่ม.......รหัส ...................... ชื่อ-สกุล....................................................................

3)  ปัญหาอย่างหนึ่งในการใช้ Natural Gas for Vehicle (NGV) ในรถยนต์คือ ตัวถังบรรจุที่มีน้ำหนักมากและจุมวล NGV ได้น้อยต้องเติมบ่อยๆ ทั้งนี้เนื่องจากต้องบรรจุภายใต้ความดันที่สูงมาก ถังจึงต้องทำด้วยโลหะที่หนามากเพื่อความแข็งแรง

Consider for the following design:

- Maximum mass of NGV			  15  kg

- Maximum designed temperature		  50  oC

- Inside tank  volume			  40 litres

- The NGV has the following composition:

| Composition | % by Volume | M (kg/kmol) | R (kJ/kgK) |
| --- | --- | --- | --- |
| Methane, CH4 | 85 | 16 | 0.5182 |
| Ethane, C2H6 | 10 | 30 | 0.2765 |
| Nitrogen, N2 | 5 | 28 | 0.2968 |

Determine

- the mole fraction and mass fraction of each component,

- the mixture Molecular weight and Gas constant,

- the maximum pressure inside the storage tank.				(10 marks)

System: Gas mixture, Closed system

Assumption: Ideal gas

Analysis: Calculation based on 100 kmol of mixture

| Composition | % By | M | R | yi | mi | mfi |
| --- | --- | --- | --- | --- | --- | --- |
|  | volume | kJ/kmol | kJ/kmolK | kmol | kg |  |
| Methane, CH4 | 85 | 16 | 0.5182 | 0.8500 | 13.60 | 0.7556 |
| Ethane, C2H6 | 10 | 30 | 0.2765 | 0.1000 | 3.00 | 0.1667 |
| Nitrogen, N2 | 5 | 28 | 0.2968 | 0.0500 | 1.40 | 0.0777 |
|  |  |  |  | 1.0000 | 18.00 | 1.000 |

ห้องสอบ..............ที่นั่งสอบ ............กลุ่ม.......รหัส ...................... ชื่อ-สกุล....................................................................

4)  A piston-cylinder device contains 6 kg of H2 and 21 kg of N2 at 250 K and 5 MPa. Heat is now transferred to the device, and the mixture expands at constant pressure until the temperature rises to 350 K. Determine the heat transfer during this process by treating the mixture as an ideal gas and using specific heat at 300 K. (10 marks)

| System: | mixture of H2 + N2 -- Closed system |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assumptions: | Ideal gas with constant specific heat, neglect KE and PE |  |  |  |  |  |  |  |
| Analysis: | 1st law |  |  |  |  |  |  |  |
|  | 1Q2 = 1W2 +U2 - U1 |  |  |  |  |  |  |  |
| P = C : | 1W2 = PdV |  | = P(V2-V1) = P2V2 -P1V1 |  |  |  |  |  |
|  | 1Q2 = (U2 + P2V2) - (U1 + P1V1) = H2 - H1 |  |  |  |  |  |  |  |
|  | 1Q2 = Σmi(h2-h1)i |  |  | = ΣmiCpi(T2-T1)i |  |  |  |  |
| Components | mi | Cpi | T1 | T2 | mi(h2-h1)i |  |  |  |
|  | kg | kJ/kg | K | K | kJ |  |  |  |
| given | given | A-2(a) | given | given | mCp(T2-T1) |  |  |  |
| H2 | 6 | 14.31 | 250 | 350 | 8,584 |  |  |  |
| N2 | 21 | 1.04 | 250 | 350 | 2,182 |  |  |  |
| sum | 27 |  |  |  | 10,766 |  |  |  |

the heat transfer during this process,  Qin   =   H2-H1   =     10,766 kJ             Answer

ห้องสอบ..............ที่นั่งสอบ ............กลุ่ม.......รหัส ...................... ชื่อ-สกุล....................................................................

5)  Air at  1 MPa and 600oC enters a converging nozzle with a velocity of 150 m/s.  The nozzle throat area is 50 cm2 and the back pressure is 0.4 MPa.  Determine

- the stagnation temperature and stagnation pressure,

- the critical pressure and the pressure at exit (throat),

- the mass flow rate through the nozzle.

(10 marks)