Thermodynamics II           Test #4  Friday 12 September 2008 Time 1 hr

เลขที่สอบ……… Name …………………..…………………………

Examiner: Assoc.Prof.Sommai Priprem,  PhD. .________________________________________________________

1. Air at 200 kPa, 127°C, and Mach number Ma = 0.8 flows through a duct. Find the velocity, stagnation temperature and the stagnation pressure of the air.           (5 Marks)

Solution		Fluid :Air

P	=	200	kPa

T	=	127	C	=	400 K

M	=	0.8

Table A-1, A-2		at 400 K

R	=	287 J/kgK; 	k = 1.395 ;  Cp = 1013 kJ/kgK

c	=	√kRT		=	√(1.4)(287 J/kgK)(300 K)

=	400.2	m/s

V	=	Mc		=	(0.8)*(400.2 m/s)

=	320.1	m/s	Answer

To 	=	  T + V^2/2Cp		=	450.6	K

=	177.6	C	Answer

k/(k-1)  		=	3.5316

Po / P	=	(To/T)^k/(k-1)		=	0.6567

Po	=	(0.6567)*(200 kPa)	=	131.3	kPa	Answer

2. An aircraft moving in air at 300 K with a velocity of 280 m/s. Determine the speed of sound and the Mach number.        (5 Marks)

Solution					V	=	280	m/s

c	=	√kRT	;	M	=	V/c

R	=	287	J/kgK

T =	300	K	Table A-2a		k	=	1.4

c	=	√kRT	=	√(1.4)(287 J/kgK)(300 K)	=	347.2	m/s

M	=	V/c	=	(280 m/s)/(347.2 m/s)	=	0.8065	Answer

เลขที่สอบ……… Name …………………..…………………………

3. Steam flows through a device with a pressure of 0.8 MPa, a temperature of 400°C, and a velocity of 275 m/s. Determine the Mach number of the steam at this state by assuming ideal-gas behavior with k = 1.3.            (5 Marks)

Solution		Steam at 0.8 MPa and 	T	=	400	C

=	673	K

V	=	275	m/s

k	=	1.3

Table A-1		R	=	461.5	J/kgK

c	=	√kRT	=	√(1.3)(461.5 J/kgK)(673 K)

=	635.4	m/s

M	=	V/c	=	(275 m/s)/(635.4 m/s)

=	0.4328	Answer

4. Air enters a converging-diverging nozzle at 0.5 MPa with a negligible velocity. Assuming the flow to be isentropic, determine the back pressure that will result in an exit Mach number of 1.8.                                                          (5 Marks)

Solution

Pb

Po = 0.5 MPa

P*

Pe

Pc

Me = 1.8

Solution   	Fluid: Air k = 1.4

Given		Po	=	500	kPa

M	=	1.8

Table A-34		P/Po	=	0.17404

Pb   =  Pe	=	(0.17404)*(500 kPa)

=	87.02	kPa	Answer