ภาควิชาวิศวกรรมเครื่องกล

คณะวิศวกรรมศาสตร์  มหาวิทยาลัยขอนแก่น

ข้อสอปลายภาค ประจำภาคต้น ปีการศึกษา 2552

วิชา 175 232 Thermodynamics II

วันที่  19 กันยายน  2552	                     		              เวลา 09:00-12:00 น.

ผู้ออกและตรวจข้อสอบ: รศ.ดร.สมหมาย ปรีเปรม

คำสั่ง

- ให้เขียน ชื่อ-สกุล กลุ่ม และลำดับในใบเซ็นชื่อ ลงในกระดาษข้อสอบทุกแผ่น

- ข้อสอบทั้งหมดมี 5 ข้อ จำนวน 7 แผ่นรวมทั้งแผ่นนี้ ให้ทำทุกข้อ

- มีตาราง Thermodynamics แจกให้ ห้ามขีดเขียนในตารางโดยเด็ดขาด

- อนุญาตให้ใช้เครื่องคำนวณที่ไม่มีการบันทึกข้อมูลของรายวิชานี้ไว้

- ห้ามนำเอกสารใดๆออกจากห้องสอบ

- ให้ทำข้อสอบในแผ่นข้อสอบของข้อนั้นๆเท่านั้น (สามารถใช้ด้านหลังได้)

Solutions

ชื่อ-สกุล.......................................................รหัส ............................. กลุ่ม..................ลำดับในใบเซ็นชื่อ ........

| ข้อ | คะแนนเต็ม | คะแนน |
| --- | --- | --- |
| 1 | 10 |  |
| 2 | 10 |  |
| 3 | 10 |  |
| 4 | 10 |  |
| 5 | 10 |  |
| รวม | 50 |  |

1. Ethane (C2H6) is burned at a rate of 8 kg/h with 20% excess air during combustion process. Assuming complete combustion and a total pressure of 100 kPa, determine

(a) the air fuel ratio,

(b) the mass flow rate of air required, and

(c) the dew point temperature of the products.        (10 Marks)

Analysis
(a)  write the chemical reaction equation at 120% theoretical air
       Air Fuel Ratio, AF = mair/mfuel
(b)  Tdp = Tsat @ Pv (H2O)
        Ideal gas mixture: Dalton's Law:  Pi/Pm = Ni/Nm

(a) the chemical reaction equation at 100% theoretical air based on 1 kmol of C2H6 :

C2H6 +a (O2 + 3.76 N2)  -->  bCO2 + cH2O + dN2

Carbon balance:				b 	= 				2

Hydrogen balance:		c 	= 	6/2  	 	=	3

Oxegen balance:				a  	=   (2b + c)/2  	=	3.5

Nitrogen balance:			d 	= 	3.76a  		=	13.16

the chemical reaction equation at 120% theoretical air

C2H6 +1.2a (O2 + 3.76 N2)  --->   2CO2 + 3H2O + 0.2aO2 + (1.2x3.76)aN2

C2H6 +4.2 (O2 + 3.76 N2)  --->   2CO2 + 3H2O + 0.7O2 + 15.79N2

mass of air, mair 			= NM 		= (4.2*4.76 kmol)(28.97kg/kmol) 	= 579.2	kg

mass of fuel, mfuel 	= NM 		= (1 kmol)[(12x2 + 6x1)kg/kmol]	 = 30.0	kg

Air-Fuel ratio 			=   (Nm)air/(Nm)fuel  	= 	579.2/30.0

=	19.31	kg air/kg fuel	Answer

(b)	mFuel 	=	8	kg/s

mAir 	= mFuel * AF 	=	(8 kg/h)(19.31)	 =		154.4	kg/h  	Answer

(c) Consider the products composition:

Total pressure of products  =	 100 kPa

Components	N	yi	Pi		Interporation: Table A-5

kmol	 	kPa		P (kPa)	T (C)

CO2	2	0.0931	9.3067		10	45.81

H2O	3	0.1396	13.9600		13.9600	52.27

O2	0.7	0.0326	3.2573		15	53.97

N2	15.79	0.7348	73.4760

Sum	21.49	1.0000	100

Interporation at  Pv = 13.96 kPa  Tdp = 52.3 oC

Therefore, the dew point temperature of the products is  52.27	oC	Answer

Tp = 1200K

2.  Liquid propane (C3H8) enters a combustion chamber at 25°C at a rate of            1.2 kg/min where it is mixed and burned with 150 percent excess air that enters the combustion chamber at 25°C. If the combustion is complete and the exit temperature of the combustion gases is 1200 K, determine

(a) the mass flow rate of air and

(b) the rate of heat transfer from the combustion chamber.

(10 marks)

Solution: Combustion eqn based on 1 kmol flel

C3H8 (l)+12.5(O2 + 3.76 N2)  -->  CO2 + 4H2O + 7.5O2 + 47N2

AF ratio:   	mair   	=  	(12.5)(4.76)(28.97)   	=  1,723.7   	kg

mfuel	=	(3*12)+(1*8)		=       44	kg

AF ratio = mair/ mfuel	=	1,723.7/44	=39.18   kg air/kg fuel

mdot,air = mdot,fuel * AF  	=   (1.2 kg/min)(39.18kg air/kg fuel)

=	47.01 kg/min    Answer

| 1st Law: Qcv + Sum(Hr) = Sum(Hp) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reactants at 280K |  |  |  |  |  |  |
| Components | N | hf | h280 | h298 |  | Nh |
| C3H8 | 1.0 | - 118,910 |  |  | - | - 118,910.0 |
| O2 | 7.5 | - | 8,682 | 8,682 | - |  |
| N2 | 47.0 | - | 8,669 | 8,669 | - |  |
|  |  |  |  |  | Sum (Hr) | - 118,910.0 |
| Products at 1500K |  |  |  |  |  |  |
| Components | N | hf | h1200 | h298 | Deltah | Nh |
| CO2 | 3.0 | - 393,520 | 53,848 | 9,364 | 44,484 | - 1,047,108.0 |
| H2O | 4.0 | - 241,820 | 44,380 | 9,904 | 34,476 | - 829,376.0 |
| O2 | 7.5 | - | 38,447 | 8,682 | 29,765 | 223,237.5 |
| N2 | 47.0 | - | 36,777 | 8,669 | 28,108 | 1,321,076.0 |

Sum (Hp)	-        332,170.5

Q=Hp-Hr	-        213,260.5

Mfuel	             44	kg/kmol

mfuel	            1.2	kg/min

Nfuel	0.02727273	kmol/min

Qdot	    (5,816.20)	kJ/min

(96.94)	kW

(b) the rate of heat transfer from the combustion chamber is 96.94 kW   Answer

3a.  The Airbus A-340 passenger plane has a maximum takeoff weight of about 260,000 kg, a length of 64 m., a wing span of 60 m, a maximum cruising speed of 945 km/h, a seating capacity of 271 passengers, maximum cruising altitude of 14,000 m, and a maximum range of 12,000 km . The air temperature at the crusing altitude is about -60°C. Determine the Mach number of this plane for.the stated conditions.  (5 marks)

T = -60 °C

V = 945 km/hr

3b. Air enters a converging-diverging nozzle at 0.5 MPa with a negligible velocity. Assuming the flow to be isentropic, determine the back pressure that will result in an exit Mach number of 1.8.       (5 Marks)

Pb

AIR

Po = 0.5 MPa

P*

Pe

Pc

Solution   	Fluid: Air k = 1.4

Given		Po	=	500	kPa

M	=	1.8

Table A-34	P/Po	=	0.17404

Pb   =  Pe	=	(0.17404)*(500 kPa)

=	87.02	kPa	Answer

4. Air at  1 MPa and 600oC enters a converging nozzle with a velocity of 150 m/s.  Determine the mass flow rate through the nozzle of the nozzle throat area of 50 cm2 when the back pressure is 0.4 MPa.  (10 marks)

5.  Air enters a converging-diverging nozzle of a supersonic wind tunnel at 1 MPa and 37°C with a low velocity. The flow area of the test section is equal to the exit area of the nozzle, which is 0.5 m2. Calculate the pressure, temperature, velocity, and mass flow rate in the test section for a Mach number Ma = 2.

(10 marks)

6. The velocity of steam leaving the nozzle of an impulse turbine is 900 m/s and the nozzle angle is 20o. The blade velocity is 300 m/s and the blade velocity coefficient is 0.7. Calculate for the mass flow of 1 kg/s,  and symmetrical blading  ( = ).

(a) the blade inlet angle;

(b) the driving force on the wheel;

(c) the axial thrust;

(d) the power output of the turbine,

(e) the blade efficiency      (10 marks)

Solution

Given:

- V1  = 900 m/s ;   α = 20o

- VB  = 300 m/s

- kB = V2R / V1R = 0.7

Determine: , Rt , Ra , w, B

b = VB

a = V1

α

c=V1R

β

A

B

C

D

Triangle ACD: law of cosines

c2     =  a2 + b2 – 2ab cos α

V1R =  c = 626.5 m/s

Triangle ABD:  law of sines

c/sin  =  a/sin(180o-) = a/sin

sin     = (a/c)(sin) = 0.491

the blade inlet angle,  = sin-1(0.491) = 29.41o    answer

Tangential Thrust; Rt  = mdot(V1R cos β + V2R cos γ)

kB = V2R / V1R = 0.7  V2R = 0.7V1R = 0.7x626.5 = 438.5 m/s

V2

δ

VB

V2R

γ

symmetrical blading  ( = ).

Rt  = mdot (626.5 cos 29.41 + 438.5 cos 29.41)

Rt /mdot 	= 927.7   N/(kg/s)

(b) the driving force on the wheel  per 1 kg/s

Rt / mdot  = 927.7   N/(kg/s)    			answer

Axial Thrust;

Ra 	= mdot  (V2R sin γ - V1R sin β) - (P1A1 – P2A2)

Impulse turbine 	    ΔP across blade = 0 ,

Symmetry blading    A1 =A2

Ra /mdot  	= (V2R sin γ - V1R sin β)

Ra  	= (438.5 sin 29.41 - 626.5 sin 29.41)

= - 92.3  N/(kg/s)   			answer (c)

(d) the turbine power, Wdot = mdot VB RT

Wdot 	  = (1 kg/s)(300 m/s)(927.7 N/kg/s)/(1000 W/kW)

=  278.3 kW  					 answer (d)

(e) the blade efficiency, D = w/ke1 = Wdot/KE1dot

KE1dot = mdot V12/2 = (1kg/s)(900 m/s)2/(1000 W/kW) = 810 kW

B   =  (278.3 kW)/(810 kW)x100% = 68.7 %       answer (e)