ข้อ

คะแนน

1

2

รวม

5

4

3

ผู้ตวรจ

Thermodynamics II                                             Test no.2  Saturday 18 December 2010  Time 10:00-11:30

เลขที่…...Section…...ID………………….Name………….…..……………………

1. A 20x10x5 m3 room contains air at 20°C and 98 kPa at a relative humidity of 85 percent. Calculate (a) the dew point of the air, (b) the mass of water and the mass of dry air, (c) the specific humidity of the air, and (d) the enthalpy per unit mass of dry air.             	                                                                                                 (5 Marks)

@=1

1

1

1

1

1

1

2) The air in a room at a pressure of 1 atm, has a dry-bulb temperature of 32°C, and a wet-bulb temperature of 26°C. Using the psychrometric chart, determine (a) the specific humidity, (b) the enthalpy (in kJ/kg dry air), (c) the relative humidity, (d) the dew-point temperature. ( 5 Marks)

| Answer | Value | Unit |
| --- | --- | --- |
| (a) the specific humidity | 0.019 | kg/kg da. |
| (b) the enthalpy | 80.9 | kg/kg da. |
| (c) the relative humidity | 62 | % |
| (d) the dew-point temperature | 24 | oC |

Thermodynamics II                                             Test no.2  Saturday 18 December 2010  Time 10:00-11:30

เลขที่…...Section…...ID………………….Name………….…..……………………

2

2

1

ระบบ-ระเบียบ-สะอาด- 2

2

1

3) The air in problem 2) was cooled by a cooling coil to 15oC. Show the process occurred. Write the analysis and using Psychrometric chart to determine, (a) the amount of moisture removed per kg dry air, and (b) the amount of heat removed per kg dry air. (10 marks)

(b) Water mass balance

mv1 + mw, add = mv2

mw, add  =  mv2 - mv1

mw, add  = ma (w2 - w1)  ………..(1)

from Psychrometric chart:

w1 = 0.019 and w2 = 0.0107 kg/kg da

therefore,   	mw, add  	=  (1 kg da.)*( 0.0107- 0.019) kg/kg da

=  -0.0083 kg

The water removed is  0.0083 kg/kg da		Answer

(b) CV. Cooling coil: 1st Law, SSSF neglect ΔKE ΔPE and W = 0

q + h1 = h2

q 	 = h2 - h1…………………….(2)

from Psychrometric chart:

h1 = 80.9	and 	h2 = 42.0 kJ/kg da

therefore,   		q  	=  (42.0 – 80.9) kJ/kg da

=  -38.9 kJ/kg da.

The heat removed is  38.9 kJ/kg da.		Answer

Thermodynamics II                                             Test no.2  Saturday 18 December 2010  Time 10:00-11:30

เลขที่…...Section…...ID………………….Name………….…..……………………

Thermodynamics II                                             Test no.2  Saturday 18 December 2010  Time 10:00-11:30

เลขที่…...Section…...ID………………….Name………….…..……………………

10 oC, 90 %RH, 5 kg/min

35 oC, 40 %RH, 20 kg/min

1

3

2

- w3 = ?

- Tdb3=?

Φ1

Tdb1

Φ2

Tdb2

Φ3

Tdb3

1

2

ระบบ-ระเบียบ-สะอาด- 1

2

1

1

1

1

4) Two airstreams are mixed steadily and adiabatically. The first stream enters at 35°C and 40 percent relative humidity at a rate of 20 kg/min, while the second stream enters at 10°C and 90 percent relative humidity at a rate of 5 kg/min. Assuming that the mixing process occurs at a pressure of 100 kPa. Show the process on a psychromatic chart. Write the analysis,  including the detail of how each variable is to be determine, in order to determine (a) the specific humidity, (b) the dry-bulb temperature of the mixture. (10 marks)

Thermodynamics II                                             Test no.2  Saturday 18 December 2010  Time 10:00-11:30

เลขที่…...Section…...ID………………….Name………….…..……………………

5) A wet cooling tower is to cool 25 kg/s of cooling water from 40 °C to 30°C at a location where the atmospheric pressure is 96 kPa. Atmospheric air enters the tower at 20°C, and 70 percent relative humidity and leaves saturated at 35°C. Make up water is available at 20°C. Neglecting the power input to the fan. Write the analysis,  including the detail of how each variable is to be determine, in order to determine (a) the volume flow rate of air into the cooling tower and (b) the mass flow rate of the required makeup water.            (10 marks)

Warm water in

Cold water out

Air out

Air in

Make up water

3

5

1

2

4

2

2

1

ระบบ-ระเบียบ-สะอาด- 1

2

1

1