ผู้ตรวจ

...................................

ข้อ

คะแนน

1

2

รวม

No……ID……….…………Name………………….……

_________________________________________________________________

1. Steam flows steadily through a turbine. The inlet conditions of the steam are 15 MPa, 650°C, and 200 m/s, and the exit conditions are 10 kPa, 90 percent quality, and 50 m/s. The mass flow rate of the steam is 150 kg/s. Heat loss from the turbine is 15 kJ/kg steam. State any assumptions made. Show the process on a T-v diagram with respect to saturation line and determine

Wcv

Turbine

1

2

P1 = 15 MPa

T1 = 650 C

V1 = 200 m/s

m = 150 kg/s

P2 = 10 kPa

x2 = 90%

V2 = 50 m/s

ΔKE = ?

Wcv = ?

q = 15 kJ/kg

(a)  the change in kinetic energy,

(b) the power output of the turbine.

Solution

Assumption: SSSF process,  ΔPE = 0

Analysis:

ΔKE = m(ke2-ke1)	    	   (1)

1st law: q + h1 + ke1 = w + h2 + ke2

w = q + (h1-h2) - (ke1-he2)    (2)

W = mw                                (3)

Property:

State 1:	P1 = 15 MPa, T1 = 650 oC

T > Tsat --> superheat

Table A-6, 	v1 	=  	0.0268 m3/kg

h1 	=  	3,712.30  kJ/kg

State 2:	     P2 	= 	10 kPa, x2 = 90 %  --> mixture

Table A-5     vf  	= 	0.001452 m3/kg,	vg = 0.018026 m3/kg

v2 	= 	vf + x vfg   	= 0.0163686 m3/kg

hf 	= 	191.83 kJ/kg,  		hfg   = 2392.8 kJ/kg

h2 	= 	hf + xhfg   	      	= 2,345.35  kJ/kg

(a)	 ΔKE    =   m(ke2-ke1)

(ke2-ke1)     = 1/2{(200m/s) 2 - (50m/s) 2)}/1000 = -18.75	kJ/kg

m      = 150 kg/s

ΔKE     = m(ke2-ke1) = (12 kg/s)(-1.95 kJ/kg) = -2,812.50	kW

Change in kinetic energy (decreased) = 2,812.5 	kW	Answer

(b) from eqn 2	w 	= 	q + (h1-h2) - (ke1-ke2)

q 	=	-15	kJ/kg

(h1- h2)	=	3712.3 kJ/kg - 2345.35 kJ/kg  = 1,367.0	kJ/kg

ke1-ke2      =	18.75	kJ/kg

w 	= 	(-15kJ/kg) + (1,367.0kJ/kg)+( 18.75kJ/kg)

=     1,333.2	kJ/kg

W      =  mw 	=	(150 kg/s)(1333.2 kJ/kg)

=	   199,980 	 kW

Power output from the turbine is   200  MW  		Answer

ผู้ตรวจ.........................

No……ID……….……………Name ………………….…………………………

___________________________________________________________________

5. Refrigerant-12 enters the evaporator of an air conditioner at 200 kPa with a quality of 25% at a rate of 2.1 kg/min and leaves as saturated vapor at the same pressure. Air enters at 100 kPa and 28°C. It is required that the air leaves the evaporator at 16°C at 100 kPa. Assume air as an ideal gas and use constant specific heat at 300 K in the calculation. State any assumption made.  Determine

(a) the mass flow rate of the air (in kg/min) and,

(b) the volume flow rate of the air at inlet (in m3/min).

1

R-12

P1 = 200 kPa

x1 = 25%

mR = 2.1 kg/min

AIR

P3    = 100 kPa

T3   = 28 oC

ma =  ?  Va3 = ?

4

3

2

AIR

T4 = 16 oC

P4 = P3

R-12

Sat. vapor

P2 = P1

| R-12-side data and calculations |  |  |  |  | Air-side data and calculations |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| State 1 mixture |  |  |  |  | State 3 and 4 Ideal gas |  |  |  |  |
| P1 | = | 200 | kPa |  | P3 | = | 100 | kPa |  |
| x1 | = | 25% |  |  | T3 | = | 28 | C |  |
| mr | = | 2.1 | kg/min |  | T4 | = | 16 | C |  |
| From Table A-12 at 200 kPa |  |  |  |  | From table A-2 at 300K |  |  |  |  |
| hf | = | 24.57 | kJ/kg |  | Cp | = | 1.005 | kJ/kgK |  |
| hfg | = | 157.5 | kJ/kg |  | Cp(T3-T4) | = | 12.06 | kJ/kg |  |
| h1=hf+hfg | = | 63.945 | kJ/kg |  | Substitute in equation (1) and (2) |  |  |  |  |
| State 2 Saturated vapor: Table A-12 |  |  |  |  | Eqn. (1) ma | = | 20.57 | kg/min | Answer |
| P2 | = | 140 | kPa |  | R | = | 0.287 | kJ/kgK |  |
| h2=hg | = | 182.07 | kJ/kg |  | v3=RT3/P3 | = | 0.86387 | m3/kg |  |
| mr(h2-h1) | = | 248.06 | kJ/min |  | Vdot3 = mav3 | = | 17.77 | m3/min | Answer |

Sommai Priprem     August 2010