Combustion HW 3

15-48 Methane (CH4) is burned completely with the stoichiometric amount of air during a steady-flow combustion process. If both the reactants and the products are maintained at 25°C and 1 atm and the water in the products exists in the liquid form, determine the heat transfer from the combustion chamber during this process. What would your answer be if combustion were achieved with 100 percent excess air? Answer: 890,330 kJ/kmol

15-50 Liquid propane (C3H8) enters a combustion chamber at 25°C at a rate of 1.2 kg/min where it is mixed and burned with 150 percent excess air that enters the combustion chamber at 12°C. If the combustion is complete and the exit temperature of the combustion gases is 1200 K, determine (a) the mass flow rate of air and (b) the rate of heat transfer from the combustion chamber. Answers: (a) 47.1 kg/min, (b) 5,194 kJ/min

15-53 Benzene gas (C6H6) at 25°C is burned during a steady-flow combustion process with 95 percent theoretical air that enters the combustion chamber at 25°C. All the hydrogen in the fuel burns to H2O, but part of the carbon burns to CO. If the products leave at 1000 K, determine (a) the mole fraction of the CO in the products and (b) the heat transfer from the combustion chamber during this process. Answers: (a) 2.1 percent, (b) 2,112,800 kJ/kmol C6H6

15-61 A constant-volume tank contains a mixture of 1 kmol of benzene (C6H6) gas and 30 percent excess air at 25°C and 1 atm. The contents of the tank are now ignited, and all the hydrogen in the fuel burns to H2O but only 92 percent of the carbon burns to CO2, the remaining 8 percent forming CO the final temperature in the tank is 1000 K, determine the heat transfer from the combustion chamber during this process..

15-68 Acetylene gas (C2H2) at 25°C is burned during steady-flow combustion process with 30 percent excess at 27°C. It is observed that 75,000 kJ of heat is being lost from the combustion chamber to the surroundings per kmol of acetylene. Assuming combustion is complete, determine the exit temperature of the product gases. Answer: 2301